PRODUCT INFORMATION

Detaprofi Series

Universal spot removal series

pH value		Focal point of application

Detaprofi Blodex	8.5-9.5			Blood & albumen stains (food)

Detaprofi Tanex	2.0-2.5			Tannin stains (coffee, fruit)

Detaprofi Lacex	7.0-8.0			Dye, grease and wax stains

Detaprofi Medex	11.0-11.6		Drugs and ointment stains, marks from						buttons

Detaprofi Ferex	3.5-4.5			Rust and metal stains

Application

The special spotters of the Detaprofi Series can be applied as pre and post spotting agents in perc and in water.   Working with hydrocarbon solvents the spotters of the Detaprofi series are only applicable as post-spotting agents due to their bad rinsing behaviour and due to the danger of the decrease of the flashpoint.

Before using these spotting agents on sensitive garments and on textiles with low fastness to colour we recommend carrying out a seam test.

Prespotting (Perc & Water)

The spotters of the Detaprofi series should be rinsed with water and the wet area should be covered with a pre-brushing agent.   Treating sensitive garments (i.e. wool) the wet area has to be blown dry.

If the textiles, which are pretreated with spotters of the Detaprofi series, are wet cleaned or washed subsequently, it is not necessary to rinse the treated areas of course.

Pre-treated items should be cleaned or washed within 20 minutes at the latest.

Post-spotting

Detaprofi spotters should react for approx. 30 seconds on the stain, the effect is intensified by mechanical action (spatula, brush) and steam.   Rinse the treated area with water, vacuum and air dry.   All spotting agents of the Detaprofi series can be rinsed out rimless.

Comments

Special spotting agents should be used sparingly!   The new dosing nozzle of the working bottle enables an application of the product drop by drop.

Before using a further agent the previous one has to be rinsed with water thoroughly and it has to be air-dried to a large degree.

Treating stains which are capable of swelling in water, an improvement of the effect of the special spotting agents is possible by spraying water on to the spotting area.

Problems treating stains from lacquer and oil paints

Stains from the mentioned group cause problems by lacquers free of solvents as they are absolutely insoluble in cured condition.   They can only be removed by icing with chewing gum spray and subsequent brushing.

Application

Wet and after-spotting:

When used for wet and after-spotting allow the Detaprofi agents to react on the stains for about 30 seconds using spatula, spotting brush and steam to support the effect.   Rinse the treated area with water, vacuum and air dry.

0800 074 2325

www.textile-care.co.uk

sales@textile-care.co.uk